Bioenergy source: Bagasse

What is bagasse? When sugarcane is squeezed for its juice, a fibrous pulp material is left over. This material is processed into a usable form called ‘bagasse’. For each 10 tonnes of sugarcane crushed, nearly 3 tonnes of wet bagasse is produced. Bagasse is typically used to produce heat and electricity in sugar mills (this is known as cogeneration), but can also be used for a variety of other purposes such as to make paper, cattle feed and even disposable food containers. Bagasse in Australia Australia’s sugar industry has used bagasse to meet its electricity and heat requirements for over 100 years. Today, bagasse is a major contributor in the bioenergy sector - accounting for over 60 per cent of Australia’s dedicated bioenergy capacity. In Australia the crushing season generally lasts about 22 weeks. During the off-season, bagasse is unavailable so sugar mills need to use alternative fuel supplies in order to generate power year-round. Most sugar mills have traditionally used coal during the off-season, but are now considering low-emission alternatives, such as co-firing with wood pellets… Benefits for energy from bagasse Bagasse offers many unique benefits, such as: - It helps sugar mills to meet 100% of their energy needs. This is important, as sugar milling is highly energy intensive. Furthermore, sugar milling seasons often coincide with peak demand loads, so sugar mills can benefit immensely from the opportunity to sell surplus electricity to the grid at peak power rates. - The facilities needed to generate heat and electricity from bagasse are often located at or near the sugar mill. Generating heat and electricity at the point of energy demand removes the need for costly transportation of the bagasse; involves minimal transmission and distribution costs; and reduces network losses and augmentation. - Energy from bagasse generates less greenhouse gas emissions than conventional fossil-fuel generation. In addition, if bagasse were left to rot, it would break down and release greenhouse gases, particularly methane, which is 27 times more dangerous to the ozone than carbon dioxide. Adapted from: ‘Using bagasse for bioenergy’ in Bioenergy bulletin (Clean Energy Council) https://www.cleanenergycouncil.org.au/dam/cec/technologies/bioenergy/fact-sheets/Bioenergy-Fact-Sheet-Using-Bagasse-for-Bioenergy.pdf Used with permission.

Use the the text above to answer the following questions.

- How is bagasse made?

- How is bagasse used by the sugar industry?

- How is bagasse used to generate electricity?

- Bagasse is not a renewable energy source. True or false. Explain why.